Choice of our project

In selecting project, our interested is in controller side. So first choice is always a controller inside our project. Being EC engineering we want to know some point about on wireless transmition. So we decide project must be wireless. Anther point we are interested in broadcast transmition, means only one transmitter room and many receiver room. So we put in our project two receiver and only one transmitter room, and when we decide then signal is passing through particular receiver-1 and when we decide signal is passing through receiver-2 that is requirement of our project. Also all this transmittion data that are not merge with each other that we want. That is basic need to choice of our project.

BASIC CHART:-

flow of data

Above show basic flow of project. Need transmitter, Control room, and receiver No of receiver you can increase, but here we see you only two receivers for demo. Transmitter signal is pass to the control room in control room some control signal is send to the receiver -1 and receiver-2 depending upon transmitter data (Temp.). This is complete wireless ckt.

Inside, transmitter one temperature sensors , controller, and transmitter ckt.

Inside, control room receiver ckt, controller, LCD and again transmitter ckt.

Inside, receiver-1 receiver ckt, controller, and LCD, motor

Inside, receiver-2 receiver ckt, controller and LCD, motor

Below pages show you circuit diagram and part description.

Component description:-

- LM 35 (Temperature sensors)

The LM3 series are precision integrated-circuit temperature sensors, whose output voltage is linearly proportional to the Celsius (centigrade) temperature. The LM35 thus has an advantage over linear temperature sensors calibrated in ‘Kelvin’, as the user is not required to subtract a large constant voltage from its output to obtain convenient centigrade scaling. The LM35 does not required any external calibration or trimming to provide typical accuracies of ¼  C at room temperature and ¾ C over a full -35 to +150 C temperature rang. Low coast is assured by trimming and calibration at the wafer level. The LM35’s low output impedance, linear output, and precise inherent calibration make interfacing to read out or control circuitry especially easy. It can be used with single power supplies, or with plus and minus supplies. As it draws only 60 uA from its supply, it has very low self-heating, less than 0.1 C in still air. The LM35 is rated to operate over a -55 to +150 C temperature range, while the LM35C is rated for -40 to +110 C ranges. The lm35 series is available packaged in hermetic TO-46 transistor packages, while the LM35C, LM35CA, and LM35D are available in plastic TO-92 transistor package. The LM35D is also available in an 8-lead surface mount small outline package and a plastic TO-220 package.

The LM35 can be applied easily in the same way as other integrated        circuit temperature sensors. It can be glued or cemented to a surface and its temperature will be within about 0.01°C of the surface temperature. This presumes that the ambient air temperature is almost the same as the surface temperature; if the air temperature were much higher or lower than the surface temperature, the actual temperature of the LM35 die would be at an intermediate temperature between the surface temperature and the air temperature. This is expecially true for the TO-92 plastic package, where the copper leads are the principal thermal path to carry heat into the device, so its temperature might be closer to the air temperature than to the surface temperature. To minimize this problem, be sure that the wiring to the LM35, as it leaves the device, is held at the same temperature as the surface of interest. The easiest way to do this is to cover up these wires with a bead of epoxy which will insure that the leads and wires are all at the same temperature as the surface, and that the LM35 die’s temperature will not be affected by the air temperature.

The TO-46 metal package can also be soldered to a metal surface or pipe without damage. Of course, in that case the V− terminal of the circuit will be grounded to that metal. Alternatively, the LM35 can be mounted inside a sealed-end metal tube, and can then be dipped into a bath or screwedinto a threaded hole in a tank. As with any IC, the LM35 and accompanying wiring and circuits must be kept insulated and dry, to avoid leakage and corrosion. This is especially true if the circuit may operate at cold temperatures where condensation can occur. Printed-circuit coatings and varnishes such as Humiseal and epoxy paints or dips are often used to insure that moisture cannot corrode the LM35 or its connections. These devices are sometimes soldered to a small

light-weight heat fin, to decrease the thermal time constant and speed up the response in slowly-moving air. On the other hand, a small thermal mass may be added to the sensor, to give the steadiest reading despite small deviations

in the air temperature.

Below figure shows LM35CZ view and diameter..

LM35CZ is linear temperature vs. voltage characteristics. That is clearly shows below figure.

Temperature rise of lm35 related on different packet that is shown in below.

- ADC 0804

Description

The ADC0804 family are CMOS 8-Bit, successive-approximation A/D converters which use a modified potentiometric ladder and are designed to operate with the 8080A control bus via three-state outputs. These converters appear to the processor as memory locations or I/O ports, and hence no interfacing logic is required. The differential analog voltage input has good commonmode- rejection and permits offsetting the analog zero-inputvoltage value. In addition, the voltage reference input can be adjusted to allow encoding any smaller analog voltage span to the full 8 bits of resolution.

Features

• 80C48 and 80C80/85 Bus Compatible - No Interfacing

Logic Required

• Conversion Time < 100ms

• Easy Interface to Most Microprocessors

• Will Operate in a “Stand Alone” Mode

• Differential Analog Voltage Inputs

• Works with Bandgap Voltage References

• TTL Compatible Inputs and Outputs

• On-Chip Clock Generator

• 0V to 5V Analog Voltage Input Range (Single + 5V Supply)

PIN DIAGRAM OF ADC0804

CONNECTION WITH MICROCONTROLLER

ADC controlling is two done by three pin, named CS , WR and RD. for enable of ADC we should kept CS pin as a logic 0. then given a WR and RD pulse in proper sequence. When CS pin is logic 0, and WR pin make logic 0 to 1, then ADC takes a analog data from Vin pin. It takes a some time for calculate is called conversion time. Mans that if conversion time is related how much faster ADC. Normally we choose frequency 120 us

Understanding A/D Error Specs

A perfect A/D transfer characteristic (staircase wave-form) is shown in Figure 11A. The horizontal scale is analog input voltage and the particular points labeled are in steps of 1 LSB (19.53mV with 2.5V tied to the VREF/2 pin). The digital output codes which correspond to these inputs are shown as D-1, D, and D+1. For the perfect A/D, not only will center-value (A - 1, A, A + 1, . . .) analog inputs produce the correct output digital codes, but also each riser (the transitions between adjacent output codes) will be located 1/2 LSB away from each centervalue.  is shown, the risers are ideal and have no width. Correct digital output codes will be provided for a range of analog input voltages which extend 1/2 LSB from the ideal center-values. Each tread (the range of analog input voltage which provides the same digital output code) is therefore 1 LSB wide. The error curve of Figure 11B shows the worst case transfer function for the ADC0802. Here the specification guarantees

that if we apply an analog input equal to the LSB analog voltage center-value, the A/D will produce the correct digital code. Next to each transfer function is shown the corresponding error plot. Notice that the error includes the quantization uncertainty of the A/D. For example, the error at point 1 of Figure 11A is +1/2 LSB because the digital code appeared 1/2 LSB in advance of the center-value of the tread. The error plots always have a constant negative slope and the abrupt upside steps are always 1 LSB in magnitude, unless the device has missing codes.

Detailed Description

The functional diagram of the ADC0802 series of A/D converters operates on the successive approximation principle (see Application Notes AN016 and AN020 for a more detailed description of this principle). Analog switches are closed sequentially by successive-approximation logic until the analog differential input voltage [VlN(+) - VlN(-)] matches a voltage derived from a tapped resistor string across the reference voltage. The most significant bit is tested first and after 8 comparisons (64 clock cycles), an 8-bit binary code (1111 1111 = full scale) is transferred to an output latch. The normal operation proceeds as follows. On the high-to-low transition of the WR input, the internal SAR latches and the shift-register stages are reset, and the INTR output will be set high. As long as the CS input and WR input remain low, the A/D will remain in a reset state. Conversion will start from 1 to 8 clock periods after at least one of these inputs makes a lowto- high transition. After the requisite number of clock pulses to complete the conversion, the INTR pin will make a high-to-low transition. This can be used to interrupt a processor, or otherwise signal the availability of a new conversion. A RD

operation (with CS low) will clear the INTR line high again.

Digital Operation

The converter is started by having CS and WR simultaneously low. This sets the start flip-flop (F/F) and the resulting “1” level resets the 8-bit shift register, resets the Interrupt (INTR) F/F and inputs a “1” to the D flip-flop, DFF1, which is at the input end of the 8-bit shift register. Internal clock signals then transfer this “1” to the Q output of DFF1. The AND gate, G1, combines this “1” output with a clock signal to provide a reset signal to the start F/F. If the set signal is no longer present (either WR or CS is a “1”), the start F/F is reset and the 8-bit shift register then can have the “1” clocked in, which starts the conversion process. If the set signal were to still be present, this reset pulse would have no effect (both outputs of the start F/F would be at a “1” level) and the 8-bit shift register would continue to be held in the reset mode. This allows for asynchronous or wide CS and WR signals. After the “1” is clocked through the 8-bit shift register (whichcompletes the SAR operation) it appears as the input to DFF2. As soon as this “1” is output from the shift register, the AND gate, G2, causes the new digital word to transfer to the Three-State output latches. When DFF2 is subsequently clocked, the Q output makes a high-to-low transition which causes the INTR F/F to set. An inverting buffer then supplies the INTR output signal. When data is to be read, the combination of both CS and RD being low will cause the INTR F/F to be reset and the threestate output latches will be enabled to provide the 8-bit digital outputs.

Digital Control Inputs

The digital control inputs (CS, RD, and WR) meet standard TTL logic voltage levels. These signals are essentially equivalent to the standard A/D Start and Output Enable control signals, and are active low to allow an easy interface to microprocessor control busses. For non-microprocessor based applications, the CS input (pin 1) can be grounded and the standard A/D Start function obtained by an active low pulse at the WR input (pin 3). The Output Enable function is achieved by an active low pulse at the RD input (pin 2).

Analog Operation

The analog comparisons are performed by a capacitive charge summing circuit. Three capacitors (with precise ratioed values) share a common node with the input to an autozeroed comparator. The input capacitor is switched between VlN(+) and VlN(-), while two ratioed reference capacitors are switched between taps on the reference voltage divider string. The net charge corresponds to the weighted difference between the input and the current total value set by the successive approximation register. A correction is made to offset the comparison by 1/2 LSB

Clocking Option

The clock for the A/D can be derived from an external source such as the CPU clock or an external RC network can be added to provIde self-clocking. The CLK IN (pin 4) makes use of a Schmitt trigger as shown in Figure

clock circuit

Restart During a Conversion

If the A/D is restarted (CS and WR go low and return high) during a conversion, the converter is reset and a new conversion is started. The output data latch is not updated if the conversion in progress is not completed. The data from the previous conversion remain in this latch.

Continuous Conversions

In this application, the CS input is grounded and the WR input is tied to the INTR output. This WR and INTR node should be momentarily forced to logic low following a powerup cycle to insure circuit operation. See Figure

Power Supplies

Noise spikes on the V+ supply line can cause conversion errors as the comparator will respond to this noise. A low-inductance tantalum filter capacitor should be used close to the converter V+ pin, and values of 1F or greater are recommended. If an unregulated voltage is available in the system, a separate 5V voltage regulator for the converter (and other analog circuitry) will greatly reduce digital noise on the V+ supply. An lCL7663 can be used to regulate such a supply from an input as low as 5.2V.

Testing the A/D Converter

There are many degrees of complexity associated with testing an A/D converter. One of the simplest tests is to apply a known analog input voltage to the converter and use LEDs to display the resulting digital output code as shown in Figure 18. For ease of testing, the VREF/2 (pin 9) should be supplied with 2.560V and a V+ supply voltage of 5.12V should be used. This provides an LSB value of 20mV. If a full scale adjustment is to be made, an analog input voltage of 5.090V (5.120 - 11/2 LSB) should be applied to the VIN(+) pin with the VIN(-) pin grounded. The value of the VREF/2 input voltage should be adjusted until the digital output code is just changing from 1111 1110 to 1111 1111. This value of VREF/2 should then be used for all the tests. The digital-output LED display can be decoded by dividing the 8 bits into 2 hex characters, one with the 4 most-significant bits (MS) and one with the 4 least-significant bits (LS). The output is

then interpreted as a sum of fractions times the full scale voltage:

- Microcontroller:- 89C51

The 8051 microcontroller generic part number actually includes a whole family of microcontrollers  that have numbers ranging from 8031 to 8751 and a avilabele in N-channel metal oxide silicon and complementary metal oxide silicon construction in a vriety of package types. An enhance version of the 8051, the8052 also exists with its own family of variation and even includes one member that can be programmed in BASIC. 8051 40-pin DIP and direct the investigation of a particular type to the data books. This is unique features of microcontrollers:

The AT89C51 is a low-power, high-performance CMOS 8-bit microcomputer with 4K	bytes of Flash programmable and erasable read only memory (PEROM). The device	is manufactured using Atmel’s high-density nonvolatile memory technology and is compatible with the industry-standard MCS-51 instruction set and pinout. The on-chip Flash allows the program memory to be reprogrammed in-system or by a conventional nonvolatile memory programmer. By combining a versatile 8-bit CPU with Flash on a monolithic chip, the Atmel AT89C51 is a powerful microcomputer which provides a highly-flexible and cost-effective solution to many embedded control applications.

The AT89C51 provides the following standard features: 4K bytes of Flash, 128 bytes of RAM, 32 I/O lines, two 16-bit timer/counters, a five vector two-level interrupt architecture, a full duplex serial port, on-chip oscillator and clock circuitry. In addition, the AT89C51 is designed with static logic for operation down to zero frequency and supports two software selectable power saving modes. The Idle Mode stops the CPU while allowing the RAM, timer/counters, serial port and interrupt system to continue functioning. The Power-down Mode saves the RAM contents but freezes the oscillator disabling all other chip functions until the next hardware reset.

- Internal ROM and RAM

- I/O ports with programmable pins

- Timers and counters

- Serial data communication

8051 architecture consists of these specific features:

- Eight-bit CPU with registers A and B

- Sixteen-bit program counter and data pointer

- Eight-bit program status word

- Eight-bit stack pointer

- Internal ROM or EPROM(8751) of 0(8031) and 4k(8051)

- Internal RAM of 128 bytes:

- Four register banks, each containing eight registers

- Sixteen bytes, which may be addressed at the bit leve

- Eighty bytes of general-purpose data memory

- Thirty-two input/output pins arranged as four 8-bit ports:P0-P3

- TWO 16-bit timer/counters:TO  and  T1

- Full duplex serial data receiver/transmitter: SBUF

- Control registers: TCON, TMOD, SCON,PCON, IP and IE

- Two external and three internal interrupt sources

- Oscillator and clock circuits

PIN DIAGRAM

Beloew figure shows dimention in DIP packet.

BLOCK DIAGRAM:-

89C51 block diagram

Pin Description:-

VCC

Supply voltage.

GND

Ground.

Port 0

Port 0 is an 8-bit open-drain bi-directional I/O port. As an output port, each pin can sink eight TTL inputs. When 1s are written to port 0 pins, the pins can be used as high impedance inputs. Port 0 may also be configured to be the multiplexed low order address/data bus during accesses to external program and data memory. In this mode P0 has internal pullups. Port 0 also receives the code bytes during Flash programming, and outputs the code bytes during program verification. External pullups are required during program verification.

Port 1

Port 1 is an 8-bit bi-directional I/O port with internal pullups. The Port 1 output buffers can sink/source four TTL inputs. When 1s are written to Port 1 pins they are pulled high by the internal pullups and can be used as inputs. As inputs, Port 1 pins that are externally being pulled low will source current (IIL) because of the internal pullups. Port 1 also receives the low-order address bytes during Flash programming and verification.

Port 2

Port 2 is an 8-bit bi-directional I/O port with internal pull-up. The Port 2 output buffers a sink/source four TTL inputs. When 1s are written to Port 2 pins they are pulled high by he internal pull up and can be used as inputs. As inputs, port 2 pins that are externally being pulled low will source current (IIL) because of the internal pull up. Port 2 emits the high-order address byte during fetches from external program memory and during accesses to external data memory that uses 16-bit addresses (MOVX @ DPTR). In this application, it uses strong internal pull-ups when emitting 1s. During accesses to external data memory that uses 8-bit addresses (MOVX @ RI), Port 2 emits the contents of the P2 Special Function Register. Port 2 also receives the high-order address bits and some control signals during Flash programming and verification.

Port 3

Port 3 is an 8-bit bi-directional I/O port with internal pull-up. The Port 3 output buffers can sink/source four TTL inputs. When 1s are written to Port 3 pins they are pulled high by the internal pull up and can be used as inputs. As inputs, Port 3 pins that are externally being pulled low will source current (IIL) because of the pull up. Port 3 also serves the functions of various special features of the AT89C51 as listed below:

Port 3 also receives some control signals for Flash programming and verification.

RST

Reset input. A high on this pin for two machine cycles while the oscillator is running resets the device.

ALE/PROG

Address Latch Enable output pulse for latching the low byte of the address during accesses to external memory. This pin is also the program pulse input (PROG) during Flash programming.In normal operation ALE is emitted at a constant rate of 1/6 the oscillator frequency, and may be used for external timing or clocking purposes. Note, however, that one ALE pulse is skipped during each access to external Data Memory. If desired, ALE operation can be disabled by setting bit 0 of SFR location 8EH. With the bit set, ALE is active only during a MOVX or MOVC instruction. Otherwise, the pin is weakly pulled high. Setting the ALE-disable bit has no effect if the microcontroller is in external execution mode.

PSEN

Program Store Enable is the read strobe to external program memory. When the AT89C51 is executing code from external program memory, PSEN is activated twice each machine cycle, except that two PSEN activations are skipped during each access to external data memory.

EA/VPP

External Access Enable. EA must be strapped to GND in order to enable the device to fetch code from external program memory locations starting at 0000H up to FFFFH. Note, however, that if lock bit 1 is programmed, EA will be internally latched on reset. EA should be strapped to VCC for internal program executions. This pin also receives the 12-volt programming enable voltage (VPP) during Flash programming, for parts that require 12-volt VPP.

XTAL1

Input to the inverting oscillator amplifier and input to the internal clock operating circuit.

XTAL2

Output from the inverting oscillator amplifier.

Oscillator Characteristics

XTAL1 and XTAL2 are the input and output, respectively, of an inverting amplifier which can be configured for use as an on-chip oscillator, as shown in Figure 1. Either a quartz crystal or ceramic resonator may be used. To drive the device from an external clock source, XTAL2 should be left unconnected while XTAL1 is driven as shown in Figure 2. There are no requirements on the duty cycle of the external clock signal, since the input to the internal clocking circuitry is through a divide-by-two flip-flop, but minimum and maximum voltage high and low time specifications must be observed.

Idle Mode

In idle mode, the CPU puts itself to sleep while all the onchip peripherals remain active. The mode is invoked by software. The content of the on-chip RAM and all the special functions registers remain unchanged during this mode. The idle mode can be terminated by any enabled interrupt or by a hardware reset. It should be noted that when idle is terminated by a hardware reset, the device normally resumes program execution, from where it left off, up to two machine cycles before the internal reset algorithm takes control. On-chip hardware inhibits access to internal RAM in this event, but access to the port pins is not inhibited. To eliminate the possibility of an unexpected write to a port pin when Idle is terminated by reset, the instruction following the one that invokes Idle should not be one that writes to a port pin or to external memory.

Power-down Mode

In the power-down mode, the oscillator is stopped, and the instruction that invokes power-down is the last instruction executed. The on-chip RAM and Special Function Registers retain their values until the power-down mode is terminated. The only exit from power-down is a hardware reset. Reset redefines the SFRs but does not change the on-chip RAM. The reset should not be activated before VCC is restored to its normal operating level and must be held active long enough to allow the oscillator to restart and stabilize.

Programming Interface

Every code byte in the Flash array can be written and the entire array can be erased by using the appropriate combination of control signals. The write operation cycle is self timed and once initiated, will automatically time itself to completion. All major programming vendors offer worldwide support for the Atmel microcontroller series. Please contact your local programming vendor for the appropriate software revision.

above chart shows that when we want to writer a controller then single must giver to that particular pin high or low,

pin 2.6 low, pin 2.7 ,3.6  and  3.7 given +vcc voltage and ALE pin must transient with logic 1 to 0 to 1. that is shown in fig.

This ALE pin such type of transient is done with help of another controller inside controller writer.

Same faction is required for chip erase for passing required pulse and voltage to that pin shown in chart.

      - LCD:-

Frequently, an 8051 program must interact with the outside world using input and output devices that communicate directly with a human being. One of the most common devices attached to an 8051 is an LCD display. Some of the most common LCDs connected to the 8051 are 16x2 and 20x2 displays. This means 16 characters per line by 2 lines and 20 characters per line by 2 lines, respectively.

Fortunately, a very popular standard exists which allows us to communicate with the vast majority of LCDs regardless of their manufacturer. The standard is referred to as HD44780U, which refers to the controller chip which receives data from an external source (in this case, the 8051) and communicates directly with the LCD.

### 44780 BACKGROUND

The 44780 standard requires 3 control lines as well as either 4 or 8 I/O lines for the data bus. The user may select whether the LCD is to operate with a 4-bit data bus or an 8-bit data bus. If a 4-bit data bus is used the LCD will require a total of 7 data lines (3 control lines plus the 4 lines for the data bus). If an 8-bit data bus is used the LCD will require a total of 11 data lines (3 control lines plus the 8 lines for the data bus).

The three control lines are referred to as EN, RS, and RW.

The EN line is called "Enable." This control line is used to tell the LCD that you are sending it data. To send data to the LCD, your program should make sure this line is low (0) and then set the other two control lines and/or put data on the data bus. When the other lines are completely ready, bring EN high (1) and wait for the minimum amount of time delay (this varies from LCD to LCD), and end by bringing it low (0) again.

The RS line is the "Register Select" line. When RS is low (0), the data is to be treated as a command or special instruction (such as clear screen, position cursor, etc.). When RS is high (1), the data being sent is text data which should be displayed on the screen. For example, to display the letter "T" on the screen you would set RS high.

The RW line is the "Read/Write" control line. When RW is low (0), the information on the data bus is being written to the LCD. When RW is high (1), the program is effectively querying (or reading) the LCD. Only one instruction ("Get LCD status") is a read command. All others are write commands--so RW will almost always be low.

Finally, the data bus consists of 4 or 8 lines (depending on the mode of operation selected by the user). In the case of an 8-bit data bus, the lines are referred to as DB0, DB1, DB2, DB3, DB4, DB5, DB6, and DB7.

Timing diagram of EN,RS,RW shown below.

lcd pin diagram

Normally 16 pin are there in LCD , above diagram is14 pin. Pin 15 is VCC And pin 16 is GND. This two pins are nothing but backlight .

LCD operation

      - Ht-640

The 3 18 encoders are a series of CMOS LSIs for remote control system applications. They are capable of encoding 18 bits of information which consists of N address bits and 18_N data bits. Each address/data input is externally trinary programmable if bonded out. It is otherwise set floating internally. Various packages of the 3 18 encoders offer flexible combinations of programmable address/data to meet various application needs. The programmable address/ data is transmitted together with the header bits via an RF or an infrared transmission medium upon receipt of a trigger signal. The capability to select a TE trigger type or a DATA trigger type further enhances the application flexibility of the 3 18 series of encoders.

pin diagram

block diagram

Address/data programming (preset)

The status of each address/data pin can be individually preset to logic _high_, _low_, or _floating_. If a transmission enable signal is applied, the encoder scans and transmits the status of the 18 bits of address/ data serially in the order A0 to AD17 for the HT600/HT640/HT680 and A0 to D17 for the HT6187/HT6207/HT6247. There are some packaging limitations. The 18-pin DIP HT680, for example, offers four external data bits and eight external address bits. The remaining unpackaged bits or dummy codes are treated as floating for A0~AD17 or as pull-low for D12~D17. During an information transmission these bits are still located in their original position. But if the trigger signal is not applied, the chip only consumes a standby current which is less than 1_A. The address pins are usually preset to transmit data codes with particular security codes by the DIP switches or PCB wiring, while the data is selected using push buttons or electronic switches.The following figure shows an application using the HT680:

Transmission enable

For the TE trigger type of encoders, transmission is enabled by applying a high signal to the TE pin. But for the Data trigger type of encoders, it is enabled by applying a high signal to one of the data pins D12~D17.

      - HT-648

General Description

The 318 decoders are a series of CMOS LSIs for remote control system applications. They are paired with the 318 series of encoders. For proper operation a pair of encoder/decoder pair with the same number of address and data format should be selected (refer to the encoder/ decoder cross reference tables). The 318 series of decoders receives serial address and data from that series of encoders that are transmitted by a carrier using an RF or an IR transmission medium. It then compares the serial input data twice continuously with its local address. If no errors or unmatched codes are  encountered, the input data codes are decoded and then transferred to the output pins. The VT pin also goes high to indicate a valid transmission. The 318 decoders are capable of decoding 18 bits of information that consists of N bits of address and 18–N bits of data. To meet various applications they are arranged to provide a number of data pins whose range is from 0 to 8 and an address pin whose range is from 8 to 18. In addition, the 318 decoders provide various combinations of address/data number in different packages.

Operation

The 318 series of decoders provides various combinations of address and data pins in different packages. It is paired with the 318 series of encoders. The decoders receive data transmitted by the encoders and interpret the first N bits of the code period as address and the last 18–N bits as data (where N is the address code number). A signal on the DIN pin then activates the oscillator which in turns decodes the incoming address and data. The decoders will check the received address twice continuously. If all the received address codes match the contents of the decoder’s local address, the 18–N bits of data are decoded to activate the output pins, and the VT pin is set high to indicate a valid transmission. That will last until the address code is incorrect or no signal has been received. The output of the VT pin is high only when the transmission is valid.

pin diagram

block diagram

Flow chart:

.

Output type

There are 2 types of output to select from:

1. Momentary type

The data outputs follow the encoder during a valid transmission and    then reset.

2. Latch type

The data outputs follow the encoder during a valid transmission, and are then latched in this state until the next valid transmission occurs.

Timing diagram

      - Controller program:

Transmitter

org 0000h

mov p3,#0ffh		;p3 acts input

mov dptr,#0400h

lcall delay

back:clr p1.0	;wr 0

setb p1.1	;rd 1

setb p1.0	;wr 1

cpl p1.3	;led

lcall delay	;intr high

clr p1.1	;rd 0

mov a,p3

lcall conversion

sjmp back

delay:mov 05h,#0ffh

above:mov 06h,#02h

up:djnz 06h,up

djnz 05h,above

ret

org 0400h

db 00h,1h,2h,3h,4h,5h,6h,7h,8h,9h,10h,11h,12h,13h,14h,15h,16h,17h,18h,19h,20h,21h,22h,23h,24h,25h,26h,27h,28h,29h,30h,31h,32h,33h,34h,35h,36h,37h,38h,39h,40h,41h,42h,43h,44h,45h,46h,47h,48h,49h,50h,51h,52h,53h,54h,55h,56h,57h,58h,59h,60h,61h,62h,63h

org 0440h

db 64h,65h,66h,67h,68h,69h,70h,71h,72h,73h,74h,75h,76h,77h,78h,79h,80h,81h,82h,83h,84h,85h,86h,87h,88h,89h,90h,91h,92h,93h,94h,95h,96h,97h,98h,99h

conversion:movc a,@a+dptr

mov p2,a

ret

Control room :

;CONTROLL ROOM WITH TRANSMITION ENABLE

org 0000h

mov p1,#0ffh		;p1 acts input

mov p3,#1Fh             ;p3 some input

lcall initlcd

lcall delay

back:mov a,p1

lcall conversion

mov a,#80h

lcall command

mov a,01h

jb acc.7,n2

jb acc.6,n2

n1:clr p3.7

lcall data

sjmp back

n2:setb p3.7

lcall data

sjmp back

data:jb p3.0,nn1

mov a,#'A'

mov p2,a

nn1:jb p3.1,nn2

mov a,#'B'

mov p2,a

nn2:jb p3.2,nn3

mov a,#'C'

mov p2,a

nn3:jb p3.3,nn4

mov a,#'D'

mov p2,a

nn4:setb p3.4                      ;transmittion enable

lcall delay

clr p3.4                           ;transmittion disable

ret

delay:mov 05h,#0fh

above:mov 06h,#0fh

up:djnz 06h,up

djnz 05h,above

ret

conversion:mov 01h,a

anl a,#0f0h

rr a

rr a

rr a

rr a

orl a,#30h          ;higer order

lcall display

mov a,01h

anl a,#0fh

orl a,#30h          ;lower order

lcall display

ret

initlcd:mov a,#38h

lcall command

lcall delay

mov a,#0eh

lcall command

lcall delay

mov a,#01h

lcall command

lcall delay

mov a,#06h

lcall command

lcall delay

mov a,#80h

lcall command

lcall delay

ret

command:mov p0,a

clr p3.5	;rs=0

setb p3.6       ;e=1

lcall delay

clr p3.6	;e=0

ret

display:mov p0,a

setb p3.5       ;rs=1

setb p3.6       ;e=1

lcall delay

clr p3.6        ;e=0

ret

receiver -1

org 0000h

mov p1,#0ffh		;p1 acts input

mov dptr,#500h

lcall initlcd

lcall delay

mov 03h,#01h

back:lcall delay

mov 02,#12h             ;indication of full speed

mov a,p1

jz below

dec 03h

djnz 03,hh

mov a,#01h

lcall command

lcall delay

hh:inc 03h               ;inc for one time clr

clr p3.7

lcall display             ;from controll room

mov a,#80h

lcall command

lcall delay

sjmp back

delay:mov 05h,#0fh

above:mov 06h,#0fh

up:djnz 06h,up

djnz 05h,above

ret

below:setb p3.7

movc a,@a+dptr

lcall display

inc dptr

mov a,#00h

djnz 02,below

mov dptr,#500h

mov 03,#01h           ;for clr disply

mov a,#80h            ;start first

lcall command

lcall delay

sjmp back

initlcd:mov a,#38h

lcall command

lcall delay

mov a,#0eh

lcall command

lcall delay

mov a,#01h

lcall command

lcall delay

mov a,#06h

lcall command

lcall delay

mov a,#80h

lcall command

lcall delay

ret

command:mov p2,a

clr p3.5	;rs=0

setb p3.6       ;e=1

lcall delay

clr p3.6	;e=0

ret

display:mov p2,a

setb p3.5       ;rs=1

setb p3.6       ;e=1

lcall delay

clr p3.6        ;e=0

ret

org 500h

db ' ',' ','F','U','L','L',' ','S','P','E','E','D'

Receiver-2

org 0000h

mov p1,#0ffh		;p1 acts input

mov dptr,#500h

lcall initlcd

lcall delay

mov 03h,#01h

back:lcall delay

mov 02,#12h             ;indication of full speed

mov a,p1

jz below

dec 03h

djnz 03,hh

mov a,#01h

lcall command

lcall delay

hh:inc 03h               ;inc for one time clr

clr p3.7

lcall display             ;from controll room

mov a,#80h

lcall command

lcall delay

sjmp back

delay:mov 05h,#0fh

above:mov 06h,#0fh

up:djnz 06h,up

djnz 05h,above

ret

below:setb p3.7

movc a,@a+dptr

lcall display

inc dptr

mov a,#00h

djnz 02,below

mov dptr,#500h

mov 03,#01h           ;for clr disply

mov a,#80h            ;start first

lcall command

lcall delay

sjmp back

initlcd:mov a,#38h

lcall command

lcall delay

mov a,#0eh

lcall command

lcall delay

mov a,#01h

lcall command

lcall delay

mov a,#06h

lcall command

lcall delay

mov a,#80h

lcall command

lcall delay

ret

command:mov p2,a

clr p3.5	;rs=0

setb p3.6       ;e=1

lcall delay

clr p3.6	;e=0

ret

display:mov p2,a

setb p3.5       ;rs=1

setb p3.6       ;e=1

lcall delay

clr p3.6        ;e=0

ret

org 500h

db ' ',' ','F','U','L','L',' ','S','P','E','E','D'

Working:

NOW, time for working…inside the circuit behavior that we design.

First using LM35CZ that is temperature measurement, we measure temperature of environment. It voltate is proposnal to the C . so when temperature is increase voltage of LM35 is increase that is also shown temperature voltage figure in component study. LM35CZ output in mv. When 1 C temperature is change then it output change to 10 mv. For example when temperature value is 0 C then output of temperature sensors is 0 v. but when environment temperature is increase to 1 C then output of temperature sensor is 10 mv. LM35CZ sensitivity is 1 C means 1 C temperature must change for changing output voltage at pin 2.

This output voltage of LM35CZ is input to the ADC0804. this is a 8 bit analog to digital conveter. Here input volate of ADC is analog temperature value so we must first convert into digital for diplay ,valuable transmittion , and input of controller. This 8 bit ADC  is working under the controll singal applied to it in PIN RD, WR . ADC  conversion time is depended upon clock frequency. Means if clock frequency is more ADC work faster. But using ADC0804 clock frequency is not more then 120 us. That is given in datasheet. So chooce a component of oscillator that clock frequency is nearly or below to 120 us. Now proper given to pulse of RD and WR ADC work that is given in component description.When WR=1,RD=1 then ADC take input voltage form analog voltage and start conversion into digital, after conversion time it has a digital value in output pin D0-D7.when conversion time is finished it gives as intrupt pin low. This input voltage   digital data is given to that table that clearly identified.

♣ Table 1 Temperature v. Vout of the ADC804

| Temp. (C) | Vin (mV) | Vout (D7-D0) |
| --- | --- | --- |
| 0 | 0 | 0000 0000 |
| 1 | 10 | 0000 0001 |
| 2 | 20 | 0000 0010 |
| 3 | 30 | 0000 0011 |
| 10 | 100 | 0000 1010 |
| 30 | 300 | 0001 1110 |

Now , this digital signal is given to the microcontroller. Now micorocontroller job is convert this digital data into actually valid display for LCD. For example Means 30 C valid output of ADC is 0001 1110 this is 1Eh in hex value, but we want to convert into valuable BCD code. So inside controller this input is converted into BCD code and converted into 30h. this value is transmitted and take out this value of BCD in port 2 of microcontroller. This is parallel data, we know that for transmit a long range of data, data must be in serial. So now we convert this data into serial. And then transmitted.

HT-640 is conveted this data into serial and gives output in PIN 8. in pin 8 now sequence is change. Fist transmitted pilot signal, then sinc. signal then address signal and then data is tranmitted, that also shows in component study of HT-640. now for long rang of transmittion we choose a 434 MHz generater and signal is given to input of this generator for proper wireless transmiition in long range. Here antenna is required. That is nothing but car kitchen antenna available in market. Put the antenna 3-4 cm in antenna pin of the transmitter.

In control room this transmitted signal is catch up and display the LCD for current temperature. So first convert the incoming data into valid LCD data display data. For example if temperature is 10 C then convert into 30 and 31 for proper display in LCD .

Now control room microcontroller continue check the temperature value. If temperature value is <40 C then tranmit singal is receiver-1 using changing address, and when temperature value is >40 C then transmit singal is receiver-2.also you can communicate the receiver for tranmmitted word ‘A’,’B’,C’,’D in receiver 1 or 2. here this tranmitted data in control room is taken form port 2. and then given to tranmiter circuit.

In receiver circuit this valuable transmitted data ‘A’,B’,’C’,’D’ is display on LCD. And according to control signal that tranmitted in control room that is dectected in receiver microcontroller and then speed of then particular  receiver is decrease.